ECA18 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO3 SCENARIO-BASED ASSIGNMENT

Design and Performance Evaluation of an Integrated Photodetection, Imaging, Biomedical Sensing and Display System

# Stage 1 – Interpret the Application Requirements

The proposed optoelectronic system combines biomedical sensing, automotive LiDAR, image sensing, UV detection, face recognition, photovoltaic power generation and advanced displays. It needs high responsivity, high sensitivity, low noise, sufficient bandwidth, good linearity, low power consumption and reliable integration.

High responsivity converts received optical power into a useful photocurrent.

High sensitivity and low NEP help detect weak optical signals.

High bandwidth and short response time are important for LiDAR and fast imaging.

CMOS/CCD sensors must provide suitable image quality, speed and power consumption.

Flexible PV can provide auxiliary power in wearable applications.

OLED, MicroLED and quantum-dot displays are selected based on brightness, efficiency, lifetime and flexibility.

The complete system must be reliable, compact and thermally stable.

Application Requirement Statement: The system should accurately detect weak optical signals, provide high-speed LiDAR detection, support biomedical and imaging applications, use power efficiently and integrate suitable PV and display technologies.

# Stage 2 – Photodetector Configuration

## 1. Responsivity

Responsivity is the parameter that converts received optical power into photocurrent.

R = Ip / P, therefore Ip = R × P

## 2. PPG photocurrent variation

Given: R = 0.60 A/W and ΔP = 25 μW = 25 × 10⁻⁶ W

ΔI = R × ΔP = 0.60 × 25 × 10⁻⁶ = 15 × 10⁻⁶ A

Answer: ΔI = 15 μA. PPG detects changes in reflected light caused by changes in blood volume. The signal can be processed to estimate pulse rate.

## 3. PIN and APD comparison

| Parameter | PIN photodiode | APD photodiode |
| --- | --- | --- |
| Internal gain | No internal multiplication | Avalanche multiplication provides gain |
| Sensitivity | Good for normal light levels | Very high for weak signals |
| Noise | Generally lower | Higher because of avalanche excess noise |
| Bias voltage | Lower | Higher |
| Complexity | Simple and easy to use | More complex bias and control |
| Applications | PPG and general sensing | LiDAR and low-light detection |

## 4. LiDAR photocurrent

Given: P = 3 μW = 3 × 10⁻⁶ W; R = 0.80 A/W

Ip = R × P = 0.80 × 3 × 10⁻⁶ = 2.4 × 10⁻⁶ A

Answer: LiDAR photocurrent = 2.4 μA.

## 5. Other detector parameters

APD gain: Avalanche multiplication increases photocurrent and helps detect weak signals, but also increases noise and requires a higher bias voltage.

Dark current: Current flowing without incident light; it adds noise and reduces weak-signal accuracy.

Quantum efficiency (QE): QE = (collected electrons / incident photons) × 100%. Higher QE means a greater fraction of incident photons produces collected charge.

Noise Equivalent Power (NEP): The optical input power that gives a signal equal to the noise in a specified bandwidth. Lower NEP means better sensitivity.

Detectivity: A measure of weak-signal detection ability. Higher detectivity is better.

Linearity: Photocurrent should be proportional to optical power over the operating range. It improves measurement accuracy.

Design Decision 1: I recommend a PIN photodiode for PPG and an APD for LiDAR and low-light detection because PIN devices have low noise and simple operation, while APDs provide internal gain and high sensitivity.

# Stage 3 – Noise and SNR Strategy (15 Marks)

SNR compares signal power with noise power.

Given: Signal = −22 dBm; noise = −37 dBm

SNR(dB) = Signal power (dBm) − Noise power (dBm) = −22 − (−37) = 15 dB

Linear SNR = 10^(15/10) = 31.62. Answer: SNR = 15 dB, or about 31.62 as a linear power ratio.

Shot noise occurs due to the discrete nature of electric charge. Dark-current noise is caused by current flowing in darkness. Avalanche noise is caused by the random multiplication process in an APD.

At low optical power, shot noise and dark-current noise can become important. APDs also have avalanche excess noise. The dominant source depends on detector parameters and operating conditions.

If optical power decreases while dark current stays constant, the signal current decreases and SNR usually becomes worse. Increasing bandwidth can improve speed but also increases integrated noise.

SNR can be improved by selecting suitable gain, using optical filters, reducing dark current, cooling when appropriate, optimizing bandwidth and improving optical coupling.

Design Decision 2: I recommend a low-noise detector, optical filtering and optimized receiver bandwidth. APD gain should be kept at a suitable level because excessive gain increases avalanche noise.

# Stage 4 – Response Time and Bandwidth (10 Marks)

Given transit time tr = 20 ps = 20 × 10⁻¹² s; approximate bandwidth f ≈ 0.44 / tr

f ≈ 0.44 / (20 × 10⁻¹²) = 2.2 × 10¹⁰ Hz

Answer: Approximate transit-time bandwidth = 22 GHz.

Carrier transit time is the time taken by generated carriers to cross the detector. A shorter transit time supports higher bandwidth. The RC time constant also limits speed: a larger resistance or capacitance increases RC delay and reduces bandwidth. A larger detector area improves optical collection but generally increases capacitance. Therefore, detector area, coupling, sensitivity and speed must be balanced.

Design Decision 3: I recommend a fast APD or suitable high-speed PIN receiver with low capacitance and optimized readout circuitry for LiDAR. The 22 GHz value is the transit-time estimate only; actual system bandwidth may be lower due to RC and circuit limits.

# Stage 5 – PV, Image Sensors and Displays (15 Marks)

## A. Solar photovoltaic analysis

Given: Voc = 0.60 V, Isc = 5 A, fill factor FF = 0.75

Maximum power Pmax = Voc × Isc × FF

Pmax = 0.60 × 5 × 0.75 = 2.25 W

Answer: Maximum PV power = 2.25 W, based on the given values.

Voc is the voltage at zero output current. Isc is the current when output voltage is zero. Fill factor measures how close the I–V curve is to an ideal rectangular shape. PV efficiency is Pmax divided by incident optical power. Flexible PV devices are lightweight and can bend to fit wearables, but may have lower efficiency, durability concerns and packaging challenges.

## B. CMOS and CCD comparison

| Parameter | CMOS | CCD |
| --- | --- | --- |
| Architecture | Amplification/readout often integrated at pixel or column level | Charge is transferred across the chip to a readout node |
| Power | Usually lower | Usually higher |
| Speed | Fast readout and easy integration | Can provide high image quality but readout may be slower |
| Integration | Well suited to SoC integration | Less direct integration with processing circuits |
| Pixel scaling | Well suited to compact, low-power sensors | Scaling and readout complexity can be challenging |
| Applications | Face recognition, phones, automotive cameras | Scientific and specialized imaging |

Smaller pixels allow more pixels in the same area, but collect fewer photons per pixel and can reduce low-light performance. Pixel scaling must balance resolution, noise, dynamic range and optical quality.

## C. Emerging display comparison

| Feature | OLED | MicroLED | Quantum-dot display |
| --- | --- | --- | --- |
| Light generation | Organic materials emit light | Microscopic inorganic LEDs emit light | Quantum dots convert light to selected colours; many designs use a backlight |
| Brightness | High | Very high potential | High, depending on design |
| Contrast | Excellent; pixels can turn off | Excellent | Good to excellent depending on backlight architecture |
| Efficiency/lifetime | Good, but organic materials can age | High potential efficiency and long lifetime | Good colour efficiency; depends on display architecture |
| Flexibility | Suitable for foldable displays | MicroLED transfer and flexible integration are challenging | Usually depends on the underlying panel/backlight |
| AR/VR/MR | Compact, high contrast | Promising for very bright compact displays | Good colour performance |

Design Decision 4: I recommend flexible PV for wearable auxiliary power, a CMOS image sensor for face recognition and automotive integration, and OLED for compact foldable displays. MicroLED is a strong option where very high brightness and long lifetime are more important than cost and manufacturing complexity.

# Stage 6 – Biomedical and Industrial Applications (10 Marks)

## 1. Wearable PPG

An LED illuminates the skin and a photodiode measures reflected light. Blood-volume changes alter the received light, producing a small AC component. A PIN photodiode with suitable responsivity, low-noise amplification, filtering and signal processing is suitable. Low power consumption and stable skin contact are important.

## 2. NIR spectroscopy

Near-infrared light is directed through or reflected from biological material. Different substances absorb and scatter light differently. A detector with good NIR responsivity, suitable optical filters and signal processing measures the returned intensity. Calibration is needed before estimating material properties.

## 3. Automotive LiDAR

LiDAR sends optical pulses and measures the returning light. The time delay is used to estimate distance. A sensitive APD or other suitable fast detector, high bandwidth, narrow optical filtering and low-noise receiver improve ranging performance.

## 4. Face recognition

A CMOS camera captures facial images. NIR illumination can help in low-light or controlled illumination conditions. Resolution, pixel sensitivity, image processing speed and power consumption affect performance.

## 5. UV sensing

UV sensing requires a detector responsive to the required ultraviolet wavelength range and with low response to unwanted visible or infrared light. Optical filters, calibration and suitable packaging improve reliable UV measurement.

Design Decision 5: I recommend PIN-based PPG sensing, an NIR-sensitive detector for spectroscopy, an APD-based high-speed LiDAR receiver, a CMOS camera for face recognition and a UV-optimized photodetector for ultraviolet sensing.

# Stage 7 – MATLAB/SCILAB Simulation and Verification (5 Marks)

The following MATLAB/SCILAB-style code plots photocurrent versus optical power for a detector responsivity of 0.60 A/W and calculates the given bandwidth and SNR. This is a simple theoretical model; detailed noise simulation requires detector dark current, temperature, bandwidth and APD excess-noise parameters.

% Responsivity and bandwidth model
clear; clc; close all;
R = 0.60;                 % responsivity in A/W
P_uW = 0:5:100;           % optical power in micro-watts
P_W = P_uW * 1e-6;        % convert to watts
I_A = R .* P_W;           % photocurrent in amperes

plot(P_uW, I_A*1e6, 'o-');
xlabel('Optical Power (micro-W)');
ylabel('Photocurrent (micro-A)');
title('Photocurrent vs Optical Power');
grid on;

% Given calculations
deltaP = 25e-6;
I_PPG = 0.60 * deltaP;
I_LiDAR = 0.80 * 3e-6;
tr = 20e-12;
f = 0.44 / tr;
SNR_dB = -22 - (-37);
SNR_linear = 10^(SNR_dB/10);
Pmax = 0.60 * 5 * 0.75;

fprintf('PPG current variation = %.2f micro-A\n', I_PPG*1e6);
fprintf('LiDAR photocurrent = %.2f micro-A\n', I_LiDAR*1e6);
fprintf('Transit-time bandwidth = %.2f GHz\n', f/1e9);
fprintf('SNR = %.2f dB, linear ratio = %.2f\n', SNR_dB, SNR_linear);
fprintf('PV maximum power = %.2f W\n', Pmax);

Expected calculated outputs: PPG current variation = 15 μA; LiDAR photocurrent = 2.4 μA; transit-time bandwidth = 22 GHz; SNR = 15 dB (linear ratio 31.62); PV maximum power = 2.25 W.

Interpretation: The photocurrent plot should be linear because I = R × P. The bandwidth is the theoretical transit-time limit and may be reduced by detector capacitance and receiver circuitry. The noise model should be extended with actual device parameters for a complete noise simulation.

# Final System Verification

| Requirement | Calculated / selected result | Status |
| --- | --- | --- |
| PPG photocurrent variation | 15 μA | Meets calculation |
| LiDAR photocurrent | 2.4 μA | Meets calculation |
| Responsivity | 0.60 A/W for PPG; 0.80 A/W for LiDAR | Given values used |
| Quantum efficiency | Not numerically calculable without wavelength | Needs wavelength |
| NEP / detectivity | Definitions given; no numerical detector data supplied | Needs detector data |
| Low-light SNR | 15 dB; linear ratio 31.62 | Calculated |
| Noise performance | Filtering, low dark current, optimized bandwidth and suitable gain | Design recommendation |
| High-speed bandwidth | 22 GHz transit-time estimate | RC limit also applies |
| PV maximum power | 2.25 W | Calculated |
| Image sensor | CMOS | Selected |
| Display | OLED for foldable/compact use | Selected |
| PPG / NIR | PIN-based PPG; NIR-sensitive detector for spectroscopy | Selected |
| LiDAR / automotive | High-speed APD receiver; CMOS camera for vehicle imaging | Selected |
| Face recognition | CMOS image sensor with suitable illumination | Selected |
| UV sensing | UV-optimized detector and optical filtering | Selected |
| MATLAB/SCILAB | Theoretical photocurrent plot and calculations | Code provided |

Note: Quantum efficiency, NEP and detectivity cannot be calculated numerically from the assignment data alone because wavelength, noise current, detector area and other detector-specific values are not supplied.

# Final System Design – Integrated Optoelectronic Platform (10 Marks)

Proposed block diagram (text form):

Optical Sources (visible LED / NIR LED / LiDAR laser / UV source) → Target or Sample → Optical Filters and Lens → Application-Specific Detector (PIN / APD / CMOS image sensor / UV detector) → Low-Noise Amplifier and Signal Conditioning → ADC and Processing Unit → Output (PPG/NIR information, LiDAR distance, face recognition, UV level) → OLED Display. Flexible PV → Power Management → Auxiliary power supply.

Final parameter/design table:

| Subsystem | Selected configuration | Reason |
| --- | --- | --- |
| PPG | PIN photodiode, R = 0.60 A/W | Low noise and suitable sensitivity |
| LiDAR | APD, R = 0.80 A/W before multiplication | Internal gain for weak returns and fast detection |
| Noise control | Optical filter, low-dark-current detector, optimized bandwidth | Improves weak-signal reliability |
| Detector speed | 20 ps transit time; approximately 22 GHz limit | Suitable theoretical speed; RC must be checked |
| PV | Flexible PV for wearable auxiliary supply | Lightweight integration |
| Imaging | CMOS | Low power and SoC integration |
| Display | OLED | High contrast and foldable form factor |
| Biomedical | PPG and NIR sensing | Non-invasive optical measurement |
| Automotive | APD LiDAR plus CMOS camera | Distance measurement and image capture |
| Additional sensing | CMOS/NIR face recognition and UV-optimized detector | Application-specific response |

# Final Engineering Decision

YES, I would recommend developing and testing this integrated optoelectronic platform as a prototype. The calculated PPG current variation is 15 μA, LiDAR photocurrent is 2.4 μA, SNR is 15 dB, transit-time bandwidth is approximately 22 GHz and PV maximum power is 2.25 W. PIN photodiodes are suitable for low-noise PPG, while APDs are suitable for weak LiDAR returns. CMOS offers good integration and low power, and OLED is suitable for compact foldable displays.

However, I would not deploy it as a finished product until detector NEP, detectivity, quantum efficiency, noise performance, optical safety, thermal behaviour, reliability and full MATLAB/SCILAB noise simulations are verified with real component specifications.

# Conclusion

In this assignment, I analysed photodetectors, noise, SNR, detector bandwidth, photovoltaic power, image sensors, display technologies and application-specific sensing. The calculated values support the proposed design at a theoretical level. PIN and APD detectors, CMOS imaging, flexible PV and OLED display technologies provide a practical starting configuration. Further device-specific simulation and experimental testing are required before final deployment.

# References

1. ECA18 – Optoelectronics and Optical Communication, CO3 Scenario-Based Assignment (provided assignment sheet).

2. Standard photodetector relations: responsivity, quantum efficiency, noise-equivalent power and detectivity, as covered in optoelectronics course materials.

3. MATLAB or GNU Octave documentation for plotting and numerical calculations.