ECA18 – OPTOELECTRONICS AND OPTICAL COMMUNICATION

CO5 – CASE STUDY BASED ASSIGNMENT

DWDM METRO LINK COMMISSIONING

From Field Measurements to a Standards-Based Upgrade Decision

Student Name: Keshava Reddy                                                                  Register Number: 192412514

Department / Year: BE.ECE 3RD YEAR                                                       Date: 13:10:2026

Executive Summary

This case study diagnoses a 60 km ITU-T G.652.D metro fibre link carrying eight 10 Gb/s NRZ DWDM channels. It evaluates measured link loss, receiver margin, dispersion, PMD, OTDR events, eye/Q/BER performance, quantum limits and three upgrade options for 1.6 Tb/s. The main findings are that the as-built loss is 22.52 dB, the 1.20 dB reflective patch-panel connector is the highest-priority repair, the mechanical splice is the second priority, and Channel 5 has a channel-specific transmitter/ER problem. Option P (16 × 100G DP-QPSK) is the preferred upgrade candidate on OSNR tolerance and energy relative to Option Q/R, subject to repaired-link verification and actual transceiver/grid compatibility.

# Stage 1 – Interpret Measurement and Performance Requirements

The link must support 60 km over existing G.652.D fibre without re-laying the fibre, maintain at least 3 dB receiver margin, meet BER ≤ 10⁻¹² (Q ≥ 7.03), use the ITU-T G.694.1 frequency grid for the upgrade and improve capacity from 80 Gb/s to 1.6 Tb/s.

| Instrument | Quantity validated | Use in this case |
| --- | --- | --- |
| Optical power meter | Absolute channel power and insertion loss | Check received power and margin before/after repairs |
| Optical spectrum analyser (OSA) | Channel spectrum, OSNR and spectral occupancy | Verify OSNR and grid after upgrade |
| OTDR | Event location, reflectance, event loss and fibre attenuation | Locate bad connector/mechanical splice |
| Eye / BER tester | Eye opening, jitter, Q and BER | Diagnose Channel 5 and confirm recovery |
| Dispersion / PMD analyzer | Chromatic dispersion and differential group delay | Validate CD/PMD against receiver tolerance |

Performance Requirement Statement: After repair, the 60 km link must have total loss low enough to deliver at least −21 dBm at the receiver (−24 dBm sensitivity plus 3 dB margin), meet the BER target of 10⁻¹², achieve Q ≥ 7.03 and ER ≥ 8.2 dB for the specified sensitivity, maintain acceptable OSNR, and support a verified 1.6 Tb/s upgrade on the existing fibre pair.

# Stage 2 – Link Budget, Dispersion and OSNR

## 2.1 Fibre and as-built loss

Fibre attenuation = 60 km × 0.21 dB/km = 12.60 dB

Connector loss = 3 × 0.50 dB = 1.50 dB

Fusion splice loss = 12 × 0.05 dB = 0.60 dB

Mechanical splice (event 2) = 0.62 dB

Event 3 connector loss = 1.20 dB; event 4 loss unresolved and not double-counted

As-built total loss = 12.60 + 1.50 + 0.60 + 0.62 + 1.20 = 16.52 dB

The supplied Stage 6 table states an as-built value of 22.52 dB. The event-by-event data provided on page 2 sum to 16.52 dB when event 4 is not separately counted because its loss is unresolved. To respect the assignment’s stated Stage 6 value, this report treats 22.52 dB as the declared measured as-built total and flags a 6.00 dB discrepancy that must be reconciled against the power-meter/OTDR records before final sign-off.

Design loss allowance = 60 × 0.21 + 4 × 0.5 + 13 × 0.05 = 15.25 dB

The Stage 6 target of ≤21.25 dB appears to include an additional 6 dB engineering allowance above the calculated 15.25 dB design loss. The stated 22.52 dB is 1.27 dB above the Stage 6 target.

## 2.2 Received power and margin

Launch power is +1 dBm/channel and demultiplexer loss is 6 dB. Using the declared 22.52 dB link loss:

P_rx = +1 − 22.52 − 6.0 = −27.52 dBm/channel

Margin vs. −24 dBm sensitivity = −27.52 − (−24) = −3.52 dB

The link is 6.52 dB below the required −21 dBm received-power level (sensitivity plus 3 dB margin). If the event-by-event 16.52 dB loss were used instead, received power would be −21.52 dBm, giving 2.48 dB margin over sensitivity but still 0.52 dB short of the 3 dB design margin.

## 2.3 Extinction ratio and dispersion penalties

Using the supplied relation δ = 5 log₁₀[1 + 2π(B·D·L·σλ)²], assume B = 10 Gb/s, D = 17 ps/(nm·km), L = 60 km and σλ = 0.03 nm.

B·D·L·σλ = 10¹⁰ × 17 × 60 × 0.03 × 10⁻¹² = 0.306

δ_CD = 5 log₁₀[1 + 2π(0.306)²] ≈ 1.14 dB

For a simplified extinction-ratio penalty estimate, use ER penalty ≈ −10 log₁₀(1 − 10^(−ER/10)). This is an approximation for comparing the channels, not a replacement for receiver-specific characterization.

Channel 3 ER = 9.0 dB → penalty ≈ 0.58 dB

Channel 5 ER = 4.5 dB → penalty ≈ 1.75 dB

Using declared total loss and subtracting the estimated CD and ER penalties:

Channel 3 effective power ≈ −27.52 − 1.14 − 0.58 = −29.24 dBm

Channel 5 effective power ≈ −27.52 − 1.14 − 1.75 = −30.41 dBm

These are power-equivalent penalty estimates; they should not be mistaken for direct power-meter readings. The main failure is the insufficient link margin, worsened on Channel 5 by low ER.

## 2.4 OSNR and PMD

The assignment gives OSNR = 58 + P_in − NF, with P_in = −6 dBm/channel at the booster EDFA input and NF = 5 dB.

OSNR ≈ 58 − 6 − 5 = 47 dB (0.1 nm)

This simplified value exceeds the legacy system’s stated 20 dB OSNR requirement by 27 dB. Therefore, in the stated simplified model, received power/penalties are the limiting factors rather than amplifier OSNR. The actual end-to-end OSNR must still be measured because amplifier cascade and reference-bandwidth details are not fully specified.

PMD DGD = 0.1 ps/√km × √60 ≈ 0.775 ps

10 Gb/s bit period = 100 ps; 10% tolerance = 10 ps

The estimated 0.775 ps PMD is well below the 10 ps tolerance.

Design Decision 1: Repair the excess loss first. The declared 22.52 dB loss produces −27.52 dBm before penalties and fails the 3 dB margin requirement. OSNR appears adequate in the supplied simplified formula; raising launch power alone should not substitute for fixing the bad connector and mechanical splice.

# Stage 3 – OTDR, Splice and Connector Strategy

## 3.1 OTDR pulse and spatial resolution

Spatial pulse length is approximately cτ/(2n), with c = 2.998×10⁸ m/s and n = 1.4682.

10 ns → 1.02 m; 100 ns → 10.21 m; 1 μs → 102.1 m

| Pulse | Dynamic range | EDZ / ADZ | Use |
| --- | --- | --- | --- |
| 10 ns | 20 dB | 1 m / 4 m | Best for resolving nearby events, but limited range |
| 100 ns | 27 dB | 5 m / 15 m | Good compromise for this 60 km link and 9 m connector separation |
| 1 μs | 33 dB | 40 m / 100 m | Long reach/dynamic range, but cannot resolve events 9 m apart |

Use a 100 ns pulse for the main 60 km trace if its 27 dB dynamic range exceeds one-way trace loss plus 6 dB. Since declared total link loss is 22.52 dB, required range is 28.52 dB, so the 100 ns setting is marginally insufficient under the stated criterion. Use 1 μs for far-end visibility, then perform a short-range/launch-fibre test with 10 ns or 100 ns for local event resolution. The 10 ns range may not reach the far end reliably.

Events 3 and 4 are 9 m apart. A 10 ns pulse has an approximate 1.02 m spatial pulse length and 1 m event dead zone, so it can resolve the events if the test arrangement and reflections are controlled. The 100 ns pulse has 5 m event dead zone, so separation is borderline; the 1 μs pulse cannot resolve them. Because event 4 loss is not resolved, use a short pulse and suitable launch/receive fibres, and confirm insertion loss with a power meter.

## 3.2 Event diagnosis and repair ranking

| Event | Interpretation | Priority / action |
| --- | --- | --- |
| 1: 0.05 dB non-reflective at 12 km | Low-loss splice-like event; consistent with a good fusion splice | Low priority; document and retain |
| 2: 0.62 dB, −34 dB at 31.50 km | Excessive loss and moderate reflectance at mechanical splice | Priority 2: replace mechanical splice with fusion splice; retest both directions |
| 3: 1.20 dB, −22 dB at 59.90 km | High-loss, strongly reflective LC/UPC patch-panel connector; likely dirty, damaged, poorly mated or misaligned | Priority 1: inspect/clean, inspect end face, remate or replace connector/adapter/patch cord |
| 4: reflectance −40 dB at 59.909 km; loss unresolved | Second connector 9 m away on same patch cord | Inspect/clean and replace patch cord if needed; verify separately with short-pulse OTDR and power meter |

Fusion splicing normally provides low loss and low reflectance; a mechanical splice is more vulnerable to alignment and interface reflection. For a data-centre patch panel, LC/UPC is compact and common. If the interface and equipment permit angled physical contact, LC/APC can reduce reflection, but UPC and APC connectors must never be mated together. Connector type must match installed transceivers and patch panels.

Round-trip delay t = 2nL/c

At 12.00 km: t ≈ 117.6 μs; at 31.50 km: t ≈ 308.5 μs; at 59.90 km: t ≈ 586.6 μs

For group-index error ±0.002, approximate distance uncertainty is ΔL ≈ L(Δn/n) = L(0.002/1.4682). At 12 km: ±16.3 m; at 31.50 km: ±42.9 m; at 59.90 km: ±81.5 m. These are scale estimates; actual OTDR event location accuracy also depends on instrument calibration, pulse width and fibre parameters.

Design Decision 2: Repair event 3 first because its 1.20 dB loss and −22 dB reflectance are the worst observed event and directly affect the receiver-side patch panel. Repair event 2 next by replacing the mechanical splice with fusion splicing. Clean/replace the event 4 patch cord if needed. Verify with bidirectional OTDR traces and end-to-end power-meter measurements.

# Stage 4 – Eye Diagram, BER and Quantum Limit

## 4.1 BER from Q-factor

Use BER = 0.5 erfc(Q/√2).

Channel 3, Q = 7.6 → BER ≈ 1.5 × 10⁻¹⁴ (meets 10⁻¹²)

Channel 5, Q = 4.7 → BER ≈ 1.3 × 10⁻⁶ (fails 10⁻¹²)

Required Q is 7.03. Channel 3 is above the target; Channel 5 is well below it.

## 4.2 Total jitter and eye width

TJ = DJ + 14.07 × RJ

Channel 3: TJ = 0.18 + 14.07(0.012) = 0.34884 UI; eye width ≈ 1 − TJ = 0.651 UI

Channel 5: TJ = 0.30 + 14.07(0.022) = 0.60954 UI; eye width ≈ 1 − TJ = 0.390 UI

Channel 5 has substantially higher deterministic and random jitter, leaving a narrower horizontal timing opening. The reduced ER from 9.0 dB to 4.5 dB reduces vertical eye opening and noise margin. Check transmitter bias/modulation, laser, driver, connector cleanliness and receiver settings; replace or reconfigure the Channel 5 transmitter if ER cannot be restored above 8.2 dB.

## 4.3 Quantum-limit calculations

For BER = 10⁻¹², the ideal direct-detection relation is BER = 0.5 exp(−N), hence N = ln(0.5/BER).

N = ln(0.5 × 10¹²) ≈ 26.94 photons/bit

At 1550 nm, photon energy Eγ = hc/λ ≈ 1.282×10⁻¹⁹ J. At 10 Gb/s, ideal optical power is:

P = N × Eγ × Rb ≈ 26.94 × 1.282×10⁻¹⁹ × 10¹⁰ = 3.45×10⁻⁸ W

P(dBm) ≈ −44.6 dBm; gap from −24 dBm ≈ 19.4 dB

For coherent PSK, using the assignment’s formulae and target BER 10⁻¹²:

Homodyne: 0.5 erfc(√(2N)) = 10⁻¹² → N ≈ 6.1 photons/bit

Heterodyne: 0.5 erfc(√N) = 10⁻¹² → N ≈ 12.2 photons/bit

The corresponding ideal powers at 10 Gb/s are approximately −51.1 dBm (homodyne) and −48.1 dBm (heterodyne), using the same 1550 nm photon energy. These idealized quantum limits exclude practical losses, detector noise, implementation penalties and FEC effects; they are not real receiver sensitivity specifications.

Design Decision 3: Channel 5 is not acceptable. Its BER estimate is about 1.3×10⁻⁶, Q = 4.7 and ER = 4.5 dB all fail the stated requirements. Correct the channel-specific transmitter/driver/ER issue and re-test until Q ≥ 7.03, BER ≤ 10⁻¹² and ER ≥ 8.2 dB.

# Stage 5 – Integrated WDM Upgrade and Energy Strategy

## 5.1 G.694.1 channel-grid calculation

Frequency at n = +3 on 50 GHz grid: f = 193.1 THz + 3(0.05 THz) = 193.25 THz

Wavelength λ = c/f ≈ 299,792.458 / 193,250 nm ≈ 1551.71 nm

Option Q uses 32 GBd with roll-off α = 0.1. Approximate Nyquist occupied bandwidth is Rs(1+α) = 32 × 1.1 = 35.2 GHz. With 12.5 GHz flex-grid slots, required slots = ceil(35.2/12.5) = 3 slots, or 37.5 GHz assigned spectrum per channel.

## 5.2 Capacity, spectrum, OSNR and module power

| Option | Capacity | Approx. spectrum | OSNR requirement | Module power | Aggregate module power |
| --- | --- | --- | --- | --- | --- |
| P: 16×100G DP-QPSK | 1.6 Tb/s | 16 × 50 GHz = 800 GHz | 12.5 dB | 22 W each | 352 W |
| Q: 8×200G DP-16QAM | 1.6 Tb/s | 8 × 37.5 GHz = 300 GHz assigned (35.2 GHz occupied each) | 19.5 dB | 20 W each | 160 W |
| R: 4×400G Si-photonic | 1.6 Tb/s | 4 × 75 GHz = 300 GHz | 26 dB | 15 W each | 60 W |
| Legacy 10G SFP+ | 80 Gb/s total | 8 × 100 GHz = 800 GHz | Not specified | 3.5 W each | 28 W |

Payload spectral efficiency = aggregate payload / assigned spectrum. Option P: 1.6 Tb/s / 800 GHz = 2.0 bit/s/Hz. Option Q: 1.6 Tb/s / 300 GHz ≈ 5.33 bit/s/Hz assigned-grid efficiency (or 1.6 Tb/s / 281.6 GHz ≈ 5.68 bit/s/Hz using occupied bandwidth). Option R: 1.6 Tb/s / 300 GHz ≈ 5.33 bit/s/Hz.

Using the simplified repaired-link OSNR estimate of 47 dB from Stage 2, margins are: P = 47 − 12.5 = 34.5 dB; Q = 47 − 19.5 = 27.5 dB; R = 47 − 26 = 21 dB. These are indicative only; actual OSNR must be measured after repairs and across the full upgrade.

## 5.3 Energy per bit

Energy per bit = aggregate module power / aggregate bit rate

Option P: 352 W / 1.6×10¹² bit/s = 220 pJ/bit

Option Q: 160 W / 1.6×10¹² bit/s = 100 pJ/bit

Option R: 60 W / 1.6×10¹² bit/s = 37.5 pJ/bit

Legacy: 28 W / 80×10⁹ bit/s = 350 pJ/bit

Option R has the lowest module energy per bit and compact silicon-photonic integration, but its 26 dB OSNR requirement is the highest and leaves the smallest OSNR margin. Option Q uses less spectrum than P and consumes less module power, but DP-16QAM requires higher OSNR than DP-QPSK. Option P has the greatest spectral use and module power, but the most tolerant OSNR requirement.

Recommendation: choose Option Q if the repaired link can demonstrate OSNR ≥ 19.5 dB with adequate engineering margin, because it reaches 1.6 Tb/s using only 300 GHz assigned spectrum and 100 pJ/bit module energy. If measured OSNR or nonlinear margin is inadequate, Option P is the more conservative fallback; Option R is preferred only if its higher OSNR requirement is proven and lowest energy is the dominant objective.

Repair and verification must precede upgrade. After repair, repeat OTDR, optical power, OSA/OSNR, eye/Q/BER and CD/PMD measurements. After the upgrade, repeat per-channel power and OSNR, verify all carrier frequencies/slot widths, and check the worst-channel BER and module power.

Design Decision 4: I recommend Option Q as the balanced capacity/spectrum/energy choice, conditional on measured OSNR and transceiver compatibility. It uses 8 × 200G DP-16QAM, 12.5 GHz flex-grid slots (3 slots per channel), 300 GHz total assigned spectrum and an estimated 100 pJ/bit module energy. If its OSNR margin cannot be validated, use Option P as the conservative alternative.

# Stage 6 – Verification Table

| Requirement | Target | Result / evidence | Status |
| --- | --- | --- | --- |
| Total optical link loss | ≤ 21.25 dB | Source declares 22.52 dB; event-sum from listed components gives 16.52 dB | Reconcile 6 dB discrepancy; repair event 3/2 |
| Received power | ≥ −21 dBm | Using 22.52 dB loss: −27.52 dBm before penalties | Fail; needs ≥6.52 dB improvement |
| System margin | ≥ 3 dB | −3.52 dB relative to −24 dBm sensitivity | Fail |
| OTDR events | No event above allowance | Event 3 = 1.20 dB/−22 dB; event 2 = 0.62 dB/−34 dB | Repair 3 first, then 2 |
| Eye / Q / ER | Q ≥ 7.03; ER ≥ 8.2 dB | Ch3 Q 7.6, ER 9 dB; Ch5 Q 4.7, ER 4.5 dB | Ch3 Q passes; Ch5 fails |
| BER | ≤ 10⁻¹² | Ch3 ≈ 1.5×10⁻¹⁴; Ch5 ≈ 1.3×10⁻⁶ | Ch5 fails |
| Upgrade capacity | 1.6 Tb/s | Options P/Q/R each nominally 1.6 Tb/s | Choose after repair/OSNR validation |
| Energy per bit | Minimize | P 220; Q 100; R 37.5; legacy 350 pJ/bit | R lowest energy; Q balanced |

# Final Diagnostic and Upgrade Workflow

Field measurements → reconcile power-meter and OTDR loss totals → repair event 3 connector → replace event 2 mechanical splice with fusion splice → inspect event 4 patch cord → bidirectional OTDR + end-to-end power verification → correct Channel 5 transmitter/ER/jitter → verify Q/BER/OSNR/CD/PMD → evaluate G.694.1 grid and module energy → deploy Option Q only after OSNR margin and BER are confirmed; otherwise use Option P if its compatibility and power budget are validated.

# Final Engineering Decision

NO — do not deploy the upgraded design yet. The current link fails the stated 3 dB receiver margin using the assignment’s declared 22.52 dB as-built loss, and Channel 5 fails the Q/BER/ER targets. Event 3’s 1.20 dB high-reflectance connector is the first repair priority, followed by the 0.62 dB mechanical splice and inspection of the adjacent patch cord. The supplied event-by-event loss sum (16.52 dB) conflicts with the assignment’s declared 22.52 dB, so the power-meter and OTDR records must be reconciled. Once loss and Channel 5 are corrected and verified, Option Q is the preferred balanced 1.6 Tb/s upgrade, provided the measured OSNR exceeds 19.5 dB with appropriate margin. Option P is the fallback for greater OSNR tolerance; Option R offers the lowest module energy but the highest OSNR requirement.

# References / Source Basis

Primary source: ECA18 – Optoelectronics and Optical Communication, CO5 Case Study Based Assignment PDF supplied with the task. Standards named in the source: ITU-T G.652.D for fibre characteristics and ITU-T G.694.1 for DWDM frequency grids. Calculations follow the formulas and measurements stated in the assignment. Final field acceptance must use calibrated OTDR, optical power meter, OSA and eye/BER test results.